### Helicopter Safety Notice: Operations in High Wind, Turbulence, and Low-Level Wind Shear Near Structures

#### 1. Operating Environment & Key Hazards

High winds, turbulence, and low-level wind shear (LLWS) create highly dynamic aerodynamic conditions for helicopter operations. When operating near buildings, towers, transmission lines, or uneven terrain, mechanical turbulence and sudden wind shifts can significantly increase workload and reduce safety margins. Turbulence near structures can introduce abrupt rotor load changes, uncommanded yaw, and difficulties maintaining hover or low-speed control. LLWS can cause rapid loss of airspeed, lift, and altitude with little visual indication.

### 2. Pre-Flight Planning & Environmental Risk Assessment

Most utility helicopter operations occur in remote or undeveloped areas without nearby weather reporting stations. Because official METAR/TAF data often does not represent the actual operating environment, pilots must rely heavily on on-site environmental risk assessments before beginning work.

Essential considerations include:

- Evaluating winds aloft, surface winds, gust spread, and mechanical turbulence indicators such as blowing dust, rotor wash behavior, tree movement, or plume drift.

- Identifying terrain-induced shear zones, venturi effects in saddles or canyons, and turbulence downwind of ridgelines, structures, and towers.

- Establishing personal and organizational weather minimums for wind speed, gust factors, and directional variability.

For human external cargo (HEC) operations, the pilot must conduct a specific on-site environmental risk assessment to determine if wind, turbulence, or shear exceed safe limits. If the environment cannot be reliably assessed, the operation must be delayed or canceled.

3. In-Flight Mitigation Strategies

- Maintain airspeed margins that prevent settling-with-power or loss of control during unexpected gusts or shear.

- Fly stabilized approach and departure profiles with predetermined escape routes.

- Avoid slow flight or hover near structures unless conditions are thoroughly assessed as stable and within limits.

- Use smooth, deliberate control inputs—avoid abrupt cyclic or collective movements in turbulent conditions.

- When hovering OGE or near obstacles, continuously monitor for uncommanded yaw, rapid power changes, or wind shifts.

### 4. Hazards Near Structures & Obstructions

Structures such as transmission towers, substations, buildings, and industrial facilities create:

- Mechanical turbulence on the downwind side

- Strong updrafts, downdrafts, and rotor wash recirculation

- Sudden shifts in wind direction when maneuvering around corners, stacks, or elevated platforms

Mitigation includes maintaining lateral and vertical separation, conducting high reconnaissance passes, and selecting approach paths that avoid downwind sides of structures.

### 5. Decision-Making & Go/No-Go

Pilots should reassess conditions continuously and be prepared to abort operations at any time. Increased caution is warranted when winds exceed aircraft, pilot or company limits, gusts are unpredictable, or turbulence is moderate or higher. HEC operations should only proceed when environmental conditions are stable, predictable, and thoroughly evaluated on site.

### Summary

Helicopter operations often take place far from official weather reporting sources. High winds, turbulence, LLWS, and environmental effects from nearby structures present significant risk factors—especially during low-level maneuvering or HEC operations. Through thorough on-site environmental assessment, conservative decision-making, and disciplined flight technique, pilots can mitigate these hazards and maintain safe operation.